Corrected instruction for ACE-120 Module 3 – Project – Water Tank Step 4

Exercise 1: Adding a Bar Graph for Water Level Monitoring:

- Navigate to Objects > Chart > Bar Graph.

- Click and drag the Bar Graph onto the interface and place it next to Water Tank 1 (as shown).

- Set Data Source for Bar Graph 1:

  - Assign the Tank 1 Water Level as the input source.

    - In the general tab of the bar graph properties under the Read section, go to the Device dropdown menu and select MODBUS TCP/IP

    - Note the address of tank 1 (should be 4x-24576), enter that under that in the Address box

    - Click on the small green plus sign (looks kind of like a bookmark) to the right of where it says Device. This will bring up a menu that says Address tags.

    - In the Address Tags menu, verify that the Address type is 4x and that the address is the same as the tank 1 (24576).

    - In Address Tags menu change the Data Format to 16-bit Unsigned (if not already selected)

  - Define minimum/maximum range (e.g., 0 - 100).

    - In the Range tab of the graph attributes set Min value as 0 and Max value as 100

    - Under Alarm Indicators set Low Limit to 5 and select a Low color of your choice

    - Under Alarm Indicators set High Limit to 90 and set a color of your choice

- Set Data Source for Bar Graph 2:

  - Assign the Tank 2 Water Level as the input source.

    - Follow the same process as for Bar Graph 1 with the exception that tank 2 in the demo should have an address of 4x24577

  - Define minimum/maximum range (e.g.,0 - 100).

    - Again, follow the same process as bar graph 1

- Customize Aesthetics:

  - Adjust color schemes, axis labels, and scale ratios.

- Click OK to finalize the graph settings.

Implementing Alarm Indicators for Critical Water Levels:

This activity does not trigger an alarm. It is just an alarm indicator. It indicates that the level requires attention for either being too low or too high.

- Navigate to Data/History > Alarm Display.

  - You will see a menu saying there are No Even (Alarm) Log exists. It will ask if you want to create one, select Yes.

  - In the Event (Alarm) Log menu select New

  - In the General tab under Read select Device then MODBUS TCP/IP in the dropdown.

  - Enter the address of tank one under Address as you did for the bar graph

- Under Condition, where it says Enable if value is: select < in the first box and enter 5 in the second box. This will trigger an alarm if the water level gets below 5%

  - In the Message tab, under Text, in the Content box type in ‘Water Level too Low’, or something similar.

- In the Sound box you can Enable sound and choose a sound if you would like

- Repeat this process to make three more alarms be selecting new

  - One Alarm when tank 1 is above 90%

  - One Alarm when tank 2 is below 5%

  - One Alarm when tank 2 is above 90%

- Select Data/History then select Alarm Display

- In the Alarm tab, it would be useful to change the Font Size to something smaller such as 12 to make the display fit. Select ok.

  - Place the Alarm display on the screen. It may be useful to resize it, especially in the vertical directional to add some blank space. This is because if two alarms trigger (from tank 1 and tank 2) you will be able to see both.

- You can now run the simulation and see the bar graphs change and alarms triggering as the tanks fill

Verify these enhancements through Offline Simulation Mode to confirm functional integration.